US Bureau of Reclamation - Klamath Project

Weekly Water Report - Un-corrected Data

If you have any questions about this report, contact Jennifer Birri at 880-2540.

UPPER KLAMATH LAKE - The surface elevation of Upper Klamath Lake as of 3/8/2009 was 4141.35 above sea

level, and its storage was 350,952 a.f.  The lake elevation was up 0.37 feet from a week earlier, and the storage

was up 28,464 a.f. from a week earlier.  The lake elevation was up 0.80 feet from a month earlier, and the storage

was up 59,781 a.f. from a month earlier.  The lake elevation was up 0.19 feet from a year earlier, and the storage

was up 14,669 a.f. from a year earlier.

CLEAR LAKE - The surface elevation of Clear Lake as of 3/8/2009 was 4523.83 above sea level, and its storage

was 86,860 a.f.  The lake elevation was down 0.83 feet from a year earlier, and the storage was down 14,340 a.f.

from a year earlier.

GERBER RESERVOIR - The surface elevation of Gerber Reservoir as of 3/8/2009 was 4822.78 above sea level, and

its storage was 50,668 a.f.  The lake elevation was up 1.49 feet from a year earlier, and the storage was up 4528 a.f.

from a year earlier.

Klamath River Report

DATE	Link River	A Canal 	Keno 	Iron Gate 	Last Year	Link River	A Canal 	Keno	Iron Gate

3/1/2009	252	0	521	1,750	3/1/2008	549	0	1030	1850

3/2/2009	255	0	779	1,760	3/2/2008	552	0	1380	1850

3/3/2009	258	0	1,010	1,770	3/3/2008	555	0	1420	1840

3/4/2009	261	0	886	1,770	3/4/2008	556	0	1280	1830

3/5/2009	260	0	983	1,780	3/5/2008	556	0	1360	1830

3/6/2009	264	0	1,000	1,770	3/6/2008	556	0	1150	1840

3/7/2009	319	0	836	1,770	3/7/2008	559	0	1070	1860

3/8/2009	351	0	835	1,770	3/8/2008	564	0	1220	1860

Upper Klamath Lake Inflow in CFS

3/1/2009	-2,605	3/1/2008	1,740

3/2/2009	2,910	3/2/2008	1,786

3/3/2009	4,964	3/3/2008	1,422

3/4/2009	4,234	3/4/2008	1,812

3/5/2009	1,073	3/5/2008	1,811

3/6/2009	3,766	3/6/2008	1,034

3/7/2009	1,486	3/7/2008	1,815

3/8/2009	605	3/8/2008	1,431