US Bureau of Reclamation - Klamath

Weekly Water Report - Un-corrected Data

If you have any questions about this report, contact Jennifer Birri at 880-2540.

UPPER KLAMATH LAKE - The surface elevation of Upper Klamath Lake as of 4/28/2008 was  4142.74 above sea level,

and its storage was 462,467 a.f.  The lake elevation was up 0.14 feet from a week earlier,  and the storage was up 19,045

a.f. from a week earlier.  The lake elevation was up 0.49 feet from a month earlier,  and the storage was up 46,127 a.f.

from a month earlier.  The lake elevation was down 0.49 feet from a year earlier,  and the storage was down 29,764 a.f.

from a year earlier.

CLEAR LAKE - The surface elevation of Clear Lake as of 4/28/2008 was 4527.38 above sea level, and its storage was

152,620 a.f.  The lake elevation was down from 1.24 feet a year earlier, and the storage was down 25,030 a.f. from a

year earlier.

GERBER RESERVOIR - The surface elevation of Gerber Reservoir as of 4/28/2008 was  4831.79 above sea level, and its

storage was 80,804 a.f.  The lake elevation was down from 0.81 feet a year earlier, and the storage was down 2,956 a.f.

from a year earlier.

Klamath River Report

DATE	Link  River	A Canal 	Keno 	Iron Gate 	Last Year	Link River	A Canal 	Keno	Iron Gate

4/21/2008	1,500	305	1,860	2,860	4/21/200	1380	360	1720	2000

7

4/22/2008	1,640	355	1,860	2,880	4/22/200	1865	370	1880	2040

7

4/23/2008	1,420	355	2,880	4/23/200	1390	365	1750	2180

7

4/24/2008	1,760	376	2,870	4/24/200	1730	395	1870	2460

7

4/25/2008	1,670	425	1,960	2,870	4/25/200	1390	425	1560	2390

7

4/26/2008	1,570	435	2,010	2,860	4/26/200	1400	485	1400	2080

7

4/27/2008	1,780	435	2,040	2,900	4/27/200	1040	545	1160	1990

7

4/28/2008	1,890	500	2,090	2,910	4/28/200	933	620	1050	1820

7

Upper Klamath Lake Inflow in CFS

4/21/2008	2,519	4/21/2007	177

4/22/2008	1,167	4/22/2007	4,972

4/23/2008	5,034	4/23/2007	2,146

4/24/2008	3,352	4/24/2007	1,734

4/25/2008	2,493	4/25/2007	2,206

4/26/2008	2,403	4/26/2007	1,885

4/27/2008	2,613	4/27/2007	1,194

4/28/2008	2,379	4/28/2007	1,944