US Bureau of Reclamation - Klamath Project

Herald and News Data Sheet - Un-corrected, June 5 – 12, 2007

If you have any questions about this report, contact Jennifer Birri at 880-2540.

UPPER KLAMATH LAKE - The surface elevation of Upper Klamath Lake as of 6/12/2007 was  4142.24

above sea level, and its storage was 404732 a.f.  The lake elevation was down 0.20 feet from a week earlier,

and the storage was down 15472 a.f. from a week earlier.  The lake elevation was down 0.97 feet from a

month earlier,  and the storage was down 75113 a.f. from a month earlier.  The lake elevation was down

0.40 feet from a year earlier,  and the storage was down 30951 a.f. from a year earlier.

CLEAR LAKE - The surface elevation of Clear Lake as of 6/12/2007 was 4527.45 above sea level, and its

storage was 154020 a.f.  The lake elevation was down from 4.42 feet a year earlier, and the storage was

down 92720 a.f. from a year earlier.

GERBER RESERVOIR - The surface elevation of Gerber Reservoir as of 6/12/2007 was  4829.65 above sea

level, and its storage was 73185 a.f.  The lake elevation was down from 4.33 feet a year earlier, and the

storage was down 15699 a.f. from a year earlier.

Klamath River Report

DATE	Link  River	A Canal 	Keno 	Iron Gate 	Last Year	Link River	A Canal 	Keno	Iron Gate

6/5/2007	1,770	769	1,150	1,550	6/5/2006	2480	647	2550	2940

6/6/2007	1,530	725	1,180	1,560	6/6/2006	2780	650	2550	2940

6/7/2007	1,510	695	1,180	1,560	6/7/2006	2710	672	2570	2940

6/8/2007	1,670	681	1,180	1,550	6/8/2006	2630	670	2560	3030

6/9/2007	1,660	651	1,180	1,550	6/9/2006	2840	705	2590	3000

6/10/2007	1,690	633	1,190	1,550	6/10/2006	2770	685	2610	2940

6/11/2007	1,460	638	1,190	1,560	6/11/2006	2890	685	2620	2940

6/12/2007	1,470	670	1,190	1,550	6/12/2006	2640	695	2620	2970

Upper Klamath Lake Inflow in CFS

6/5/2007	3,319	6/5/2006	3,127

6/6/2007	694	6/6/2006	2,258

6/7/2007	254	6/7/2006	1,429

6/8/2007	1,181	6/8/2006	-6,468

6/9/2007	-30	6/9/2006	1,983

6/10/2007	2,713	6/10/2006	1,112

6/11/2007	539	6/11/2006	2,404

6/12/2007	971	6/12/2006	1,384